# Electronic Details

# Electronics Block Diagram

Attatched in seperate PDF file.

# Components Table

| Components | Quantity | Purpose | Justification |
| --- | --- | --- | --- |
| ESP32 | 1 | Central microcontroller to process sensor data and control system operations. | Provides sufficient GPIO pins, supports multiple sensors simultaneously, and enables real-time control (e.g., app/BLE integration). |
| Submersible Water Pump | 1 | Circulates nutrient-rich water through the hydroponic system. | Essential for maintaining continuous water flow, ensuring proper nutrient delivery to plants. |
| 5V ,10A 1-Channel Relay Module | 1 | Switches the water pump ON/OFF using ESP32 signals. | Allows safe control of high-power devices (pump/mains) using low-voltage microcontroller output. |
| I2C Bi-Directional logic Level Converter- 4 Channel | 1 | Converts voltage levels between ESP32 (3.3V) and 5V components. | Prevents damage to ESP32 and ensures reliable communication with higher-voltage modules like the relay. |
| LDR Sensor | 3 | Detects light intensity at different levels of the structure. | Helps monitor light distribution across the system, enabling analysis of plant exposure and environmental conditions. |
| HC-SR04 | 1 | Measure water level in the reservoir. | Enables non-contact monitoring, preventing sensor damage and ensuring accurate water level detection. |
| DHT22 | 1 | Measures temperature and humidity of the environment. | Provides reliable environmental data critical for plant growth conditions. |
| Neopixel | 1 | Provides visual feedback for system status (e.g., normal, warning, error). | Enhances user interaction and allows quick, intuitive understanding of system conditions without needing an interface. |
| Power Module | 1 | Supplies regulated power to components in the system. | Ensures stable voltage distribution, preventing fluctuations that could affect sensor readings and system reliability. |